# GarbageTrack — User Manual

System: GarbageTrack GPS Fleet Tracking System v1.2.0
Prepared for: Fleet Managers, Dispatchers, and Field Technicians
Deployment: Aliaga, Nueva Ecija, Philippines

## Table of Contents

- System Overview

- Access Credentials

- Hardware Operation

- Local Maintenance Portal

- Web Dashboard — Live Map

- Web Dashboard — Route History

- Web Dashboard — Events

- Web Dashboard — Checkpoints

- Web Dashboard — Device Configuration

- Troubleshooting

- FAQ

## 1. System Overview

GarbageTrack is an autonomous, solar-powered GPS tracking system installed on garbage trucks. It continuously records the truck’s position and transmits it to a secure cloud server, which can be monitored in real time through the web dashboard.

How it works: - The device powers on automatically when connected to the solar/battery system - It acquires GPS coordinates every few seconds - Data is uploaded via the depot’s Wi-Fi when parked, or via the LTE cellular network when on the road - If there is no internet connection (dead zone), data is saved locally and uploaded automatically once connectivity returns - The web dashboard receives updates within seconds and displays the truck’s live position on a map

## 2. Access Credentials

⚠️ Important: Keep these credentials confidential. Do not share them outside your organization. Change default passwords after first use.

### 2.1 Device Wi-Fi Access Point

When the tracker is powered on, it broadcasts its own Wi-Fi network that technicians can connect to for local diagnostics.

| Field | Value |
| --- | --- |
| Network Name (SSID) | TrackLocator-Service |
| Wi-Fi Password (WPA2) | GTrack2026 |

### 2.2 Local Maintenance Portal Login

After connecting to the device Wi-Fi, open a browser and go to http://192.168.4.1

| Field | Value |
| --- | --- |
| URL | http://192.168.4.1/ |
| Username | admin |
| Password | GTrack@2026! |

The captive portal will often open automatically on your phone. If it does not, open any browser and type the URL manually.

### 2.3 Web Dashboard Login

| Field | Value |
| --- | --- |
| Dashboard URL | https://garbage-truck-gps-z59i-2wpo36hdb-dumjaes-projects.vercel.app/ |
| Authentication | Your registered Supabase administrator email and password |

Contact your system administrator to receive your dashboard login credentials. Each user has their own secure account managed via Supabase Auth.

### 2.4 Device SIM / Connectivity

| Field | Value |
| --- | --- |
| SIM Number | 09613556081 (Smart PH) |
| Network / APN | Smart PH — APN: internet |

## 3. Hardware Operation

### 3.1 Powering On

The device starts automatically when power is available from the battery/solar system. There are no physical buttons to press. Once powered:

- The LED will blink rapidly — the device is booting and connecting

- After 30–60 seconds, the LED will slow to a steady pulse — the device is tracking and uploading

- The truck’s icon will appear on the web dashboard map

### 3.2 LED Status Indicators

| LED Pattern | Meaning |
| --- | --- |
| Rapid blink (4× per second) | Booting / connecting to network |
| Slow pulse (1× per 2 seconds) | Fully operational — GPS tracking & uploading |
| Solid ON | Error — check local portal for details |
| Off | Device is unpowered or in deep failure state |

### 3.3 Powering Off

The device should remain powered at all times. It is designed to run 24/7 on solar power. If you need to cut power for maintenance, disconnect the battery terminal from the buck converter.

### 3.4 Solar Charging

The device charges its LiFePO4 battery from the 10W solar panel via an MPPT charge controller. On a clear day, the panel generates more power than the device consumes, keeping the battery fully charged. On overcast days or at night, the battery sustains operation for approximately 12–15 hours.

## 4. Local Maintenance Portal

The device acts as its own Wi-Fi hotspot, allowing technicians to connect directly without needing a separate internet connection.

### 4.1 Connecting to the Device

- On your phone or laptop, open Wi-Fi Settings

- Look for and connect to: TrackLocator-Service

- Enter the Wi-Fi password: GTrack2026

- A “Sign in to Network” notification may appear automatically — tap it to open the portal

- If the portal does not open automatically, open a browser and go to: http://192.168.4.1/

- Enter login credentials: Username admin, Password GTrack@2026!

### 4.2 Diagnostics Dashboard

Once logged in, the portal displays a live diagnostic view that refreshes every 2 seconds:

System Health Section - Uptime — How long the device has been running since last reboot - Free Heap — Available memory (should be above 100KB; if below 50KB, reboot the device) - Consecutive Failures — Number of failed upload attempts in a row (should be 0)

Network Status - Active Interface — Shows whether data is uploading via WIFI or LTE - WiFi SSID / IP / RSSI — Current Wi-Fi connection details and signal strength - LTE Operator / RSSI — Cellular network registration and signal strength - Internet OK — Green = backend server is reachable - Authenticated — Green = device API key is valid and accepted by the server

GPS Status - GPS Fix — Green = device has a valid satellite lock - Satellites — Number of locked satellites (aim for 6+) - Accuracy (HDOP) — Lower is better; below 2.5 is good - Speed / Coordinates — Current position and speed

Storage (Offline Queue) - Buffered Records — Number of GPS records stored locally waiting to be uploaded - Capacity — Maximum is 200 records. If this is consistently full, the device cannot reach the server

### 4.3 Configure Wi-Fi (Add a New Network)

If you move the truck to a new depot with a different Wi-Fi network:

- In the portal, click “Configure Wi-Fi”

- Enter the depot’s Wi-Fi Network Name (SSID) and Password

- Click Save

- The device will attempt to connect to the new network automatically

- Watch the portal diagnostics to confirm WiFi Connected turns green

### 4.4 Rebooting the Device

If the device appears frozen or the portal is unresponsive:

- In the portal, click the “Reboot” button

- Wait 60 seconds

- Reconnect to the TrackLocator-Service Wi-Fi

- Verify the device is tracking again

## 5. Web Dashboard — Live Map

Access: https://garbage-truck-gps-z59i-2wpo36hdb-dumjaes-projects.vercel.app/

### 5.1 Logging In

- Open the dashboard URL in any browser

- Enter your registered administrator email and password

- Click Sign In

If you have forgotten your password, contact your system administrator to reset it via Supabase Auth.

### 5.2 Reading the Live Map

The main page shows a map centered on Aliaga, Nueva Ecija with animated truck icons for each tracked vehicle.

Truck Icon States: | Icon Appearance | Meaning | |—————–|———| | Animated green pulse ring | Truck is online and actively uploading data | | Gray icon, no animation | Truck has been offline for more than 3 minutes |

Clicking a Truck Icon opens a popup showing: - Current speed (km/h) - Battery percentage - Signal strength (dBm) - GPS fix status - Last seen timestamp - Approximate address (reverse geocoded from coordinates)

### 5.3 Status Cards

Below the map are real-time metric cards for each truck:

| Card | Description |
| --- | --- |
| GPS Status | Whether the device has a valid satellite fix |
| Battery | Battery percentage (0–100%) |
| Signal | LTE signal bars |
| Last Seen | Seconds/minutes since last data received |

### 5.4 Offline Detection

If a truck stops sending data for more than 3 minutes (e.g., device powered off, SIM ejected, or in a deep dead zone with no offline storage left), its icon will turn gray and the status will change to Offline. The dispatcher should investigate.

## 6. Web Dashboard — Route History

Navigate to: Sidebar → History

### 6.1 Viewing a Route

- Select a truck from the device dropdown

- Pick a start date and end date

- Click Load History

- The truck’s driven path will appear as a colored polyline on the map

- Click any dot on the route to see: time, speed, address, and coordinates at that point

### 6.2 Route Playback

Click the Play button to animate the truck’s journey:

| Control | Action |
| --- | --- |
| ▶ Play | Start animated replay |
| ⏸ Pause | Pause at current position |
| Speed (1× / 2× / 5×) | Control replay speed |
| Slider | Drag to jump to any point in the route |

## 7. Web Dashboard — Events

Navigate to: Sidebar → Events

The Events page shows a log of all system events recorded by the device and backend. Use this page to diagnose issues or audit device activity.

Common Event Types:

| Event Type | Meaning |
| --- | --- |
| upload_success | Device successfully uploaded a batch of GPS records |
| upload_fail | Device attempted to upload but the server rejected it |
| REBOOT | Device restarted (watchdog, power cycle, or manual reboot) |
| GPS_NO_FIX | Device could not obtain a GPS fix for an extended period |
| LTE_RECONNECT | Device reconnected to the LTE network after a drop |
| checkpoint_visit | Truck entered a geofenced collection checkpoint |

You can filter events by type and date range using the filter controls at the top of the page.

## 8. Web Dashboard — Checkpoints

Navigate to: Sidebar → Checkpoints

Checkpoints are virtual geofenced zones (circles on the map) that mark collection points. When a truck enters a checkpoint area, it is automatically logged.

### 8.1 Viewing Checkpoints

All defined checkpoints appear on the map as colored circles with labels.

### 8.2 Adding a New Checkpoint

- Click anywhere on the map to place a new checkpoint

- Enter the checkpoint Name (e.g., “Barangay Caalibangbangan Dropoff”)

- Set the Radius in meters (default: 50m)

- Click Save

### 8.3 Checkpoint Visit History

Click on a checkpoint circle to see a list of all truck visits — which truck arrived, at what time, and how long it stayed.

## 9. Web Dashboard — Device Configuration

Navigate to: Sidebar → Device Config

This page allows administrators to remotely adjust tracking settings for each device.

| Setting | Description |
| --- | --- |
| Recording Interval | How often GPS records are taken (seconds). Lower = more precise but more data |
| Track History | Toggle: enable or disable GPS history recording |
| Track Events | Toggle: enable or disable system event logging |
| Wi-Fi Credentials | View and update the depot Wi-Fi networks stored for this device |
| SIM Configuration | View APN and SIM settings |

Changes made here are delivered to the device on its next /config poll (every few minutes).

## 10. Troubleshooting

### Truck is not showing on the map

| Check | Action |
| --- | --- |
| Device powered? | Verify solar/battery connection. Check LED status. |
| GPS fix? | Connect to local portal → check GPS section. Move truck outdoors with clear sky view. |
| Internet connected? | Portal → check WiFi or LTE status. If no LTE, check SIM card is seated. |
| API key valid? | Portal → check “Authenticated” field. If red, contact administrator. |
| Dashboard subscription? | Refresh the browser page. |

### Portal (192.168.4.1) is not loading

| Check | Action |
| --- | --- |
| Connected to device Wi-Fi? | Confirm phone is connected to TrackLocator-Service |
| Correct URL? | Type exactly: http://192.168.4.1/ (not https) |
| Device booted? | Wait 60 seconds after power-on before trying |
| Still not loading? | Reboot the device by disconnecting and reconnecting power |

### Battery percentage is dropping

| Check | Action |
| --- | --- |
| Solar panel getting sunlight? | Ensure panel is clean and facing the sun with no shade |
| MPPT charging? | Check the charge controller LED — green = charging |
| Consumption too high? | Increase recording interval in Device Config to reduce transmissions |

### “Offline” status but truck is running

The device may be in a cellular dead zone. Check: - Events page → look for recent LTE_RECONNECT or upload_success events - Portal → Storage → check if buffered records are accumulating (device is saving data locally) - Once the truck returns to a coverage area, it will auto-upload all buffered records

## 11. Frequently Asked Questions

Q: How often does the map update?
A: The map updates every time the device sends a GPS record — typically every 5–15 seconds when moving, and every 60 seconds when stopped.

Q: What happens to data when the truck is in a dead zone?
A: The device stores up to ~200 GPS records locally in its internal flash memory. Once the truck returns to a connected area, all stored records are automatically uploaded in order with correct timestamps. No data is lost.

Q: Can I access the dashboard on my phone?
A: Yes. The dashboard is a responsive web app. Open the URL in any modern mobile browser.

Q: How do I add a new user to the dashboard?
A: Log in to the Supabase project dashboard at https://supabase.com, go to Authentication → Users, and invite a new user by email.

Q: How do I add a second truck?
A: Flash another LILYGO device with a unique DEVICE_ID and API_KEY in secrets.h, then register it in the Supabase devices table with the same API_KEY. It will automatically appear on the dashboard.

Q: Why is the battery percentage stuck at 0%?
A: The battery percentage is measured via an ADC pin. If it reads 0%, the ADC may be misconfigured or the voltage divider circuit may have a loose connection. Check the hardware wiring.

Q: The device reboots frequently. What should I do?
A: Check the Events page for REBOOT events. If the reboot reason says “watchdog”, a FreeRTOS task is hanging. This usually self-recovers. If reboots happen more than once per hour, update the firmware.

GarbageTrack v1.2.0 — September 2026
For support, contact the system administrator.