# GarbageTrack — Full-Stack Architecture

System: GarbageTrack GPS Fleet Tracking System v1.2.0
Architecture Style: IoT Edge → Cloud Backend → Web Frontend (3-tier)

## 1. System Architecture Diagram

╔══════════════════════════════════════════════════════════════════════════════╗
║                        GARBAGETRACK SYSTEM ARCHITECTURE                     ║
╠══════════════════════════════════════════════════════════════════════════════╣
║                                                                              ║
║  ┌─────────────────────────────────┐                                         ║
║  │         TIER 1: HARDWARE        │                                         ║
║  │      (On-Vehicle Edge Device)   │                                         ║
║  │                                 │                                         ║
║  │  ┌──────────────────────────┐   │                                         ║
║  │  │  LILYGO T-Call A7670E    │   │                                         ║
║  │  │  ESP32 + SIM7670 Modem   │   │                                         ║
║  │  │                          │   │                                         ║
║  │  │  FreeRTOS Tasks:         │   │                                         ║
║  │  │  ├─ TaskGPS              │   │    HTTPS POST (WiFi preferred)          ║
║  │  │  ├─ TaskUpload           │───┼─────────────────────────────────►       ║
║  │  │  ├─ TaskPortal           │   │    HTTPS POST (LTE fallback)            ║
║  │  │  ├─ TaskLED              │   │                                         ║
║  │  │  └─ TaskWatchdog         │   │                                         ║
║  │  │                          │   │                                         ║
║  │  │  Storage: NVS Flash      │   │                                         ║
║  │  │  (offline queue ~200 rec)│   │                                         ║
║  │  └──────────────────────────┘   │                                         ║
║  │                                 │                                         ║
║  │  Power: LiFePO4 12V + Solar     │                                         ║
║  │  Connectivity: WiFi + LTE (SIM) │                                         ║
║  └─────────────────────────────────┘                                         ║
║                                                                              ║
║  ┌─────────────────────────────────┐                                         ║
║  │       TIER 2: CLOUD BACKEND     │                                         ║
║  │           (Supabase)            │                                         ║
║  │                                 │    WebSocket Realtime                   ║
║  │  ┌─────────────────────────┐    │◄────────────────────────────────        ║
║  │  │   Edge Functions (Deno) │    │    (dashboard subscribes)               ║
║  │  │   ├─ /ingest            │    │                                         ║
║  │  │   ├─ /event             │    │                                         ║
║  │  │   ├─ /config            │    │                                         ║
║  │  │   ├─ /health            │    │                                         ║
║  │  │   ├─ /provision         │    │                                         ║
║  │  │   └─ /device-config     │    │                                         ║
║  │  └──────────┬──────────────┘    │                                         ║
║  │             │                   │                                         ║
║  │  ┌──────────▼──────────────┐    │                                         ║
║  │  │   PostgreSQL + PostGIS  │    │                                         ║
║  │  │   ├─ devices            │    │                                         ║
║  │  │   ├─ gps_records        │    │                                         ║
║  │  │   ├─ device_status      │    │                                         ║
║  │  │   ├─ system_events      │    │                                         ║
║  │  │   ├─ checkpoints        │    │                                         ║
║  │  │   ├─ checkpoint_visits  │    │                                         ║
║  │  │   ├─ wifi_credentials   │    │                                         ║
║  │  │   └─ sim_config         │    │                                         ║
║  │  └─────────────────────────┘    │                                         ║
║  │  Auth: Supabase Auth (JWT)       │                                         ║
║  │  Security: RLS on all tables     │                                         ║
║  └─────────────────────────────────┘                                         ║
║                                                                              ║
║  ┌─────────────────────────────────┐                                         ║
║  │     TIER 3: WEB DASHBOARD       │                                         ║
║  │   (Vite + React + TypeScript)   │                                         ║
║  │         Hosted on Vercel        │                                         ║
║  │                                 │                                         ║
║  │  Pages:                         │                                         ║
║  │  ├─ /           (LiveMap)       │                                         ║
║  │  ├─ /history    (Route Replay)  │                                         ║
║  │  ├─ /events     (Event Log)     │                                         ║
║  │  ├─ /checkpoints (Geofences)    │                                         ║
║  │  └─ /device-config (Settings)   │                                         ║
║  └─────────────────────────────────┘                                         ║
╚══════════════════════════════════════════════════════════════════════════════╝

## 2. Data Flow

### 2.1 Normal Upload (Online)

GPS Fix → Build JSON Record → Check WiFi → POST /ingest (WiFi)
                                         → POST /ingest (LTE if WiFi fails)
                                         → Supabase inserts gps_records
                                         → Trigger updates device_status
                                         → Realtime pushes to dashboard
                                         → Dashboard marker updates on map

### 2.2 Offline Buffering

GPS Fix → Build JSON Record → No Internet → Save to NVS Flash Queue
                                           → (up to ~200 records)
Network Restored → Bulk read NVS → POST /ingest (batch) → Clear NVS → Done

### 2.3 Dashboard Login Flow

User opens URL → Login page → Supabase Auth (email + password)
→ JWT issued → Stored in browser session → Dashboard loads
→ Subscribes to device_status Realtime channel → Live updates begin

## 3. Tech Stack (Detailed)

### Tier 1 — Firmware

| Component | Technology | Version |
| --- | --- | --- |
| MCU | LILYGO T-Call A7670E (ESP32-WROVER) | — |
| Framework | Arduino + FreeRTOS | ESP-IDF 5.x |
| Modem Driver | TinyGSM (A7672X) | Latest |
| HTTP | HTTPClient (WiFi) + TinyGsmClient (LTE) | — |
| JSON | ArduinoJson | v7 |
| Web Server | ESP32 WebServer | — |
| Storage | ESP-IDF NVS (Preferences) | — |
| Watchdog | esp_task_wdt | — |
| Build System | PlatformIO | — |

### Tier 2 — Backend

| Component | Technology |
| --- | --- |
| Database | PostgreSQL 15 + PostGIS (geospatial) |
| Auth | Supabase Auth (JWT, bcrypt) |
| Realtime | Supabase Realtime (WebSocket) |
| API | Supabase Edge Functions (Deno/TypeScript) |
| ORM | Supabase JS Client v2 |
| Hosting | Supabase Cloud (free tier) |

### Tier 3 — Dashboard

| Component | Technology |
| --- | --- |
| Build Tool | Vite 5 |
| Framework | React 18 + TypeScript |
| Routing | React Router v6 |
| UI Components | shadcn/ui + Radix UI |
| Maps | Leaflet.js 1.9 + React-Leaflet |
| Tile Source | OpenStreetMap (Nominatim for geocoding) |
| DB Client | @supabase/supabase-js v2 |
| Linter | Oxlint |
| Hosting | Vercel (CI/CD from GitHub) |

## 4. Database Schema

### devices

| Column | Type | Description |
| --- | --- | --- |
| id | UUID (PK) | Internal device UUID |
| device_id | TEXT UNIQUE | Human-readable ID (e.g. “TL-001”) |
| api_key | TEXT | Pre-shared key for firmware auth |
| is_active | BOOLEAN | Enable/disable device |
| track_history_enabled | BOOLEAN | Toggle GPS record storage |
| track_events_enabled | BOOLEAN | Toggle system event logging |
| recording_interval_s | INTEGER | GPS recording interval in seconds |
| created_at | TIMESTAMPTZ | — |

### gps_records

| Column | Type | Description |
| --- | --- | --- |
| id | UUID (PK) | — |
| device_id | UUID (FK → devices) | — |
| timestamp | TIMESTAMPTZ | GPS fix time |
| lat | DOUBLE | Latitude |
| lon | DOUBLE | Longitude |
| location | GEOGRAPHY(POINT) | PostGIS point (SRID 4326) |
| speed_kmh | REAL | Speed in km/h |
| heading | REAL | Direction in degrees |
| hdop | REAL | GPS accuracy indicator |
| battery_pct | INTEGER | Battery percentage |
| rssi_dbm | INTEGER | Signal strength dBm |
| gps_fix | BOOLEAN | Whether GPS had a valid fix |
| wifi_connected | BOOLEAN | Upload came via WiFi (vs LTE) |
| satellites | INTEGER | Number of GPS satellites |

### device_status

Auto-upserted by DB trigger on every gps_records insert. Stores the latest state of each device for fast dashboard queries.

| Column | Type | Description |
| --- | --- | --- |
| device_id | UUID (PK, FK) | — |
| last_seen | TIMESTAMPTZ | Timestamp of last received record |
| last_lat / last_lon | DOUBLE | Latest position |
| last_speed | REAL | Latest speed |
| battery_pct | INTEGER | Latest battery % |
| rssi_dbm | INTEGER | Latest signal strength |
| lte_connected | BOOLEAN | — |
| gps_fix | BOOLEAN | — |
| status | TEXT | online / offline |

### system_events

| Column | Type | Description |
| --- | --- | --- |
| id | UUID (PK) | — |
| device_id | UUID (FK) | — |
| event_type | TEXT | e.g. upload_success, REBOOT, GPS_NO_FIX |
| payload | JSONB | Event-specific metadata |
| created_at | TIMESTAMPTZ | — |

### checkpoints

Geofenced collection points defined by administrators.

| Column | Type | Description |
| --- | --- | --- |
| id | UUID (PK) | — |
| name | TEXT | Checkpoint name |
| lat / lon | DOUBLE | Center coordinates |
| radius_m | REAL | Geofence radius in meters |
| location | GEOGRAPHY(POINT) | PostGIS point |

### checkpoint_visits

Logged automatically when a truck enters a checkpoint geofence.

### wifi_credentials

Stores Wi-Fi networks provisioned via the captive portal for each device.

### sim_config

Stores APN and SIM configuration per device.

## 5. Edge Function API Reference

All Edge Functions are deployed at:
https://abbhqglzswbrpstndyrl.supabase.co/functions/v1/<name>

### POST /ingest

Receives GPS records from the device.

Headers:

X-Device-ID: TL-001
X-API-Key: <device api key>
Content-Type: application/json

Body:

{
  "records": [
    {
      "timestamp": "2026-09-05T13:00:00Z",
      "lat": 15.4923,
      "lon": 120.8301,
      "speed_kmh": 24.5,
      "heading": 180,
      "hdop": 1.2,
      "battery_pct": 87,
      "rssi_dbm": -75,
      "gps_fix": true,
      "wifi_connected": false,
      "satellites": 9
    }
  ]
}

Response: { "status": "ok", "accepted": 1 }

### GET /config

Returns current configuration for the device (recording intervals, thresholds).

Headers: X-Device-ID, X-API-Key

Response:

{
  "interval_moving_sec": 5,
  "interval_stopped_sec": 60,
  "interval_low_battery_sec": 300,
  "gps_accuracy_threshold_hdop": 2.5,
  "server_time": "2026-09-05T13:00:00Z"
}

### POST /event

Receives system events from the device (reboot, GPS loss, LTE reconnect, etc.).

### GET /health

Liveness check — returns { "status": "ok" }. Used by the device to verify backend reachability before attempting data upload.

### POST /provision

Registers a new device or updates its Wi-Fi credentials sent from the captive portal.

### GET /device-config

Returns the full configuration set for a device, including Wi-Fi credentials and SIM config, for the dashboard’s Device Config page.

## 6. FreeRTOS Task Map

┌────────────────────────────────────────────────────┐
│                  FreeRTOS Scheduler                 │
├──────────┬──────────┬──────────┬──────────┬────────┤
│ TaskGPS  │TaskUpload│TaskPortal│ TaskLED  │Task WDT│
│ Core 1   │ Core 1   │ Core 0   │ Core 0   │ Both   │
│ 4KB stack│ 8KB stack│ 8KB stack│ 2KB stack│ —      │
│          │          │          │          │        │
│ • Parse  │ • Read   │ • HTTP   │ • Status │ • Feed │
│   GNSS   │   NVS    │   server │   LED    │   WDT  │
│ • Update │ • POST   │ • Portal │ • Blink  │ • 30s  │
│   health │   /ingest│   UI     │   codes  │   reset│
│ • Queue  │ • Clear  │ • WiFi   │          │        │
│   record │   on OK  │   prov   │          │        │
└──────────┴──────────┴──────────┴──────────┴────────┘
     ▲           ▲
     └───────────┘
   Shared via g_health (SemaphoreHandle_t g_healthMutex)
   Modem access via g_modemMutex

## 7. Network State Machine (Firmware)

PROV_IDLE
    │ credentials submitted via portal
    ▼
PROV_CONNECTING
    │ WiFi.begin() → WL_CONNECTED
    ▼
PROV_DHCP_WAIT
    │ IP assigned
    ▼
PROV_INTERNET_CHECK   ──fail──► PROV_FAILED (FAIL_INTERNET_UNREACHABLE)
    │ HEAD /health = 200
    ▼
PROV_AUTH_CHECK       ──fail──► PROV_FAILED (FAIL_AUTH_FAILED)
    │ POST /ingest dry-run = 200
    ▼
PROV_SUCCESS (NET_TRACKING)

If WiFi fails at any stage, the firmware falls back to the LTE path:

NET_LTE_CONNECTING → NET_LTE_CONNECTED → NET_INTERNET_OK → NET_BACKEND_OK → NET_TRACKING

## 8. Security Architecture

| Layer | Mechanism |
| --- | --- |
| Transport | TLS 1.2+ (HTTPS for all cloud communication) |
| Device Authentication | X-API-Key header (per-device pre-shared key, stored in firmware secrets.h) |
| Dashboard Authentication | Supabase Auth — email/password → JWT (30-day session) |
| Database Authorization | Row-Level Security (RLS) — anon key is read-only; writes require service role key |
| Local Portal Auth | WPA2 Wi-Fi password + HTTP session token (30-minute timeout) |
| Secret Management | firmware/src/secrets.h is git-ignored; dashboard keys in .env (not committed) |

## 9. Power System Architecture

☀️ Solar Panel (10W)
        │
        ▼
  MPPT Controller
        │
        ▼
  LiFePO4 Battery (12V 10Ah)  ◄── sustains device overnight / cloudy days
        │
        ▼
  Buck Converter (12V → 5V 3A)
        │
        ▼
  LILYGO T-Call A7670E (ESP32)
  Avg draw: ~300mA @ 5V = 1.5W
  Peak (LTE transmit): ~500mA

Battery runtime without solar: ~12–15 hours
Solar surplus on sunny day: Panel generates ~40Wh; device uses ~12Wh → full charge maintained

Document generated: September 2026 | GarbageTrack v1.2.0